IOT Report

Name: Hamza Iqbal

Date: 16-09-23

Objective:

The objective of this project is to establish an IoT system utilizing an ESP32 microcontroller to collect temperature, humidity, and distance data from sensors. This data is then transmitted to a Grafana dashboard via MQTT protocol. The project also involves the use of Docker for efficient container management on a Linux server. The data transmission is facilitated through a local hotspot.

Requirements:

- ESP32 microcontroller

- Temperature and humidity sensor (e.g., DHT22)

- Ultrasonic sensor (e.g., HC-SR04)

- MQTT broker (e.g., Mosquito)

- Grafana installation

- Docker installed on the Linux server.

- Linux operating system

- Local hotspot for data transmission

Procedure:

- Hardware:

Connect the temperature and humidity sensor (DHT22) to the ESP32 as per the datasheet instructions. Connect the ultrasonic sensor (HC-SR04) to the ESP32, typically requiring two GPIO pins for trigger and echo. Ensure proper power and ground connections for both sensors and the ESP32.

- Programming:

Code :  For ultrasonic sensor.

Code: For temperature and humidity

- MQTT Broker Setup:

Install Mosquito MQTT broker on your Linux server. Configure Mosquito to listen on a specific port (e.g., 1883). Create MQTT topics for temperature, humidity, and distance data.

- Setting Up an MQTT-Based IoT Data Pipeline with Docker Containers:

- Install and Configure Mosquito MQTT Broker:

- Create a Docker container for the Mosquito MQTT broker.

- Configure MQTT broker settings and ports within the container.

- Install Grafana and Connect to MQTT Broker:

- Create a Docker container for Grafana.

- Configure Grafana to connect to the MQTT broker.

- Set up Grafana's data source to communicate with InfluxDB.

- Create a Container for InfluxDB:

- Set up a Docker container for InfluxDB to store MQTT data.

- Define InfluxDB configurations and expose necessary ports.

- Implement MQTT Data Relay Script:

- Create a Python or Node.js script to receive MQTT data from ESP32.

- Configure the script to push the data into the InfluxDB container.

- Deploy Containers with Docker Compose:

- Run the containers using Docker Compose with the docker-compose up -d command.

- Ensure all containers start without errors.

- Verify Container Status:

- Confirm the status of all containers using docker-compose ps.

- Testing and monitoring:

•	Confirm that the ESP32 is publishing data to the MQTT broker via the local hotspot.

•	Verify that data is being stored in the InfluxDB database.

•	Access the Grafana dashboard through a web browser to visualize the sensor data.

- Visualizations:

- Insights:

•	Docker provides an efficient and scalable solution for managing containers, ensuring smooth deployment and operation of the IoT system.

•	Mosquitto MQTT broker facilitates seamless communication between the ESP32 and the Grafana dashboard.

•	InfluxDB serves as a reliable database for storing and retrieving sensor data.

- Conclusion:

The project has successfully established an IoT system, integrating sensors with an ESP32 microcontroller. The data collected (temperature, humidity, and distance) is transmitted to a Grafana dashboard via MQTT protocol. Docker containerization enhances the system's scalability and manageability. The utilization of a local hotspot for data transmission ensures remote monitoring and visualization on a Linux server. This comprehensive setup lays a solid foundation for further IoT applications and data-driven.